ES 216 Signals, Systems and Networks

Programming Assignment 2

Name: Girish Chandar G

Roll No: 16110057

Discipline: Electrical Engineering

Aim:

Detection of QRS complexes and calculation of heart rates from a given ECG signals.

Theory:

Power Spectrum of ECG signal provides useful information about the QRS complex. Often the signal contains P and T waves, EMG from muscles, motion artifact from electrode and skin interface and other interferences from electro surgery equipments. Clinical Instruments such as arrhythmia monitor require real-time QRS detection. Therefore it is necessary to extract useful part of the signal, the QRS complex, from the other noises such as P and T waves, motion artifact and muscle noise.

Procedure:

The following procedures is followed to extract the necessary information from a given ECG signal

- BandPass Filter:

  - First the ECG signal is passed through a band pass filter, composed of cascaded low pass and high pass filter. Passband is 5 – 12 Hz.

  - This is done to attenuate noise.

- Derivative:

  - Then the output of the band pass signal is passed is differentiated.

  - This is done to obtain information of the slope of the signal.

- Squaring:

  - Intensifies the slope of the frequency response curve.

  - Restricts the false positives caused by the T waves

- Moving window Integration

  - Provides information of about both the slope and the width of the QRS complex

- Threshold

  - Optimum threshold is chosen to find all the peaks and no of beats accurately

Results:

Calculation:

Data       No of Beats       Beats per minute

100m.mat    	2273          	75.541914

101m.mat    	1874          	62.272271

102m.mat    	2187          	72.671978

103m.mat    	2084          	69.261249

104m.mat    	2387          	79.306307

105m.mat    	2664          	88.554982

106m.mat    	2068          	68.747758

107m.mat    	2606         	86.600528

108m.mat    	1850          	61.474240

109m.mat    	2533          	84.163615

References:

- https://ecgwaves.com/ecg-normal-p-wave-qrs-complex-st-segment-t-wave-j-point/

- Jiapu Pan et.al, “A Real Time QRS detection Algorithm”, IEEE TRANSACTIONS ON BIOMEDICAL ENGINEERING, VOL. BME-32, NO. 3,  MARCH 1985. https://ieeexplore.ieee.org/abstract/document/4122029/

- Valtino X.Alfonso, “ECG QRS Detection”, https://www.ejwoo.com/uploads/2/5/4/0/25400517/ecg_qrs_detection.pdf

- https://in.mathworks.com/help/signal/examples/peak-analysis.html